# Planha Licensed Architect Rule Review Book

Architecture residential rulebook  |  32 rules  |  Decision record template

Purpose: enable a licensed architect to confirm source interpretation, applicability, exceptions, machine logic and cross-code behavior. All decision fields are intentionally blank. This document grants no runtime, Golden, merge or deployment authority.

| Package status | AWAITING_LICENSED_ARCHITECT_REVIEW |
| --- | --- |
| Branch | feature/architecture-rulebook-qualification |
| Prepared from SHA | 4451f9953ccbd8201991e92db671ad6520fed059 |
| Allowed decisions | APPROVED ¦ APPROVED_WITH_CHANGE ¦ REJECTED ¦ NEEDS_MORE_EVIDENCE |
| Blank decision | NOT_REVIEWED |

# Reviewer instructions

Inspect the cited source and the prepared interpretation.

Verify applicability and all stated exceptions.

Verify that the machine interpretation preserves the regulation and fails closed on missing material input.

Select exactly one allowed decision for every rule.

Add comments only where needed and provide your name, professional role, license or registration identifier, and review date.

APPROVED_WITH_CHANGE requires the requested change, reason, affected logic or threshold or applicability or exception, and source basis. NEEDS_MORE_EVIDENCE requires the missing evidence, required source, materiality and risk of assumption.

## Reviewer attribution

| Reviewer name |  |
| --- | --- |
| Professional role |  |
| Professional license or registration |  |
| Review date |  |

# Summary decision sheet

| Rule ID | Short title | Source | Risk | Decision | Comment |
| --- | --- | --- | --- | --- | --- |
| ARCH-RES-CIRCULATION-001 | عرض راهروی داخلی واحد | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-STAIRS-001 | پلهٔ ساختمان مسکونی گروه ۱ تا ۳ | مبحث چهارم، الزامات عمومی ساختمان | CRITICAL |  |  |
| ARCH-RES-STAIRS-002 | پلهٔ ساختمان مسکونی گروه ۴ تا ۷ | مبحث چهارم، الزامات عمومی ساختمان | CRITICAL |  |  |
| ARCH-RES-STAIRS-003 | پلهٔ ساختمان مسکونی گروه ۸ | مبحث چهارم، الزامات عمومی ساختمان | CRITICAL |  |  |
| ARCH-RES-EGRESS-001 | بازشدن در خروج از داخل | مبحث چهارم، الزامات عمومی ساختمان | CRITICAL |  |  |
| ARCH-RES-DOOR-001 | در ورودی اتاق اقامت | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-LIVING-001 | اندازهٔ حداقل یک فضای اقامت اصلی | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-LIVING-002 | فضای اقامت واحد کوچک | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-LIVING-003 | مساحت عمومی اتاق اقامت | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-BEDROOM-001 | عرض سایر اتاق‌های اقامت مسکونی | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-KITCHEN-001 | آشپزخانهٔ مستقل یا باز برای پخت‌وپز | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-KITCHEN-002 | آشپزخانه همراه محل صرف غذا | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-KITCHEN-003 | عرض آشپزخانه | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-KITCHEN-004 | فضای کار آشپزخانه دیواری | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-KITCHEN-005 | ممنوعیت عبور الزامی از آشپزخانه | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-BATHROOM-001 | ابعاد فضای بهداشتی مسکونی معمول | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-BATHROOM-002 | ارتفاع فضای بهداشتی مسکونی | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-WC-001 | پیش‌بینی توالت فرنگی برای واحد بزرگ | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-DAYLIGHT-001 | نور و هوای آشپزخانه مستقل | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-DAYLIGHT-002 | سطح شیشه اتاق اقامت | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-DAYLIGHT-003 | پنجره با لبه بالایی کوتاه | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-DAYLIGHT-004 | عمق قابل قبول نورگیری | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-DAYLIGHT-005 | نورگیری زیر ایوان | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-DAYLIGHT-006 | حیاط بین دو تصرف مسکونی | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-DAYLIGHT-007 | حیاط مشترک آشپزخانه و اقامت | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-VENTILATION-001 | بازشوی تهویه آشپزخانه | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-VENTILATION-002 | هود به هوای آزاد | مبحث چهارم، الزامات عمومی ساختمان | HIGH |  |  |
| ARCH-RES-BALCONY-001 | فضای نیمه‌باز اختصاصی | مبحث چهارم، الزامات عمومی ساختمان | MEDIUM |  |  |
| ARCH-RES-PRIVACY-001 | دسترسی مستقل اتاق‌ها | مبحث چهارم، الزامات عمومی ساختمان | MEDIUM |  |  |
| ARCH-RES-PRIVACY-002 | دید به سرویس بهداشتی | مبحث چهارم، الزامات عمومی ساختمان | MEDIUM |  |  |
| ARCH-RES-PRIVACY-003 | دید به محل کار آشپزخانه | مبحث چهارم، الزامات عمومی ساختمان | MEDIUM |  |  |
| ARCH-RES-ACCESSIBILITY-001 | فضای بهداشتی قابل استفاده معلول | مبحث چهارم، الزامات عمومی ساختمان | CRITICAL |  |  |

# Critical and high risk review sheet

HIGH  ARCH-RES-CIRCULATION-001  عرض راهروی داخلی واحد

CRITICAL  ARCH-RES-STAIRS-001  پلهٔ ساختمان مسکونی گروه ۱ تا ۳

CRITICAL  ARCH-RES-STAIRS-002  پلهٔ ساختمان مسکونی گروه ۴ تا ۷

CRITICAL  ARCH-RES-STAIRS-003  پلهٔ ساختمان مسکونی گروه ۸

CRITICAL  ARCH-RES-EGRESS-001  بازشدن در خروج از داخل

HIGH  ARCH-RES-DOOR-001  در ورودی اتاق اقامت

HIGH  ARCH-RES-LIVING-001  اندازهٔ حداقل یک فضای اقامت اصلی

HIGH  ARCH-RES-LIVING-002  فضای اقامت واحد کوچک

HIGH  ARCH-RES-LIVING-003  مساحت عمومی اتاق اقامت

HIGH  ARCH-RES-BEDROOM-001  عرض سایر اتاق‌های اقامت مسکونی

HIGH  ARCH-RES-KITCHEN-001  آشپزخانهٔ مستقل یا باز برای پخت‌وپز

HIGH  ARCH-RES-KITCHEN-002  آشپزخانه همراه محل صرف غذا

HIGH  ARCH-RES-KITCHEN-003  عرض آشپزخانه

HIGH  ARCH-RES-KITCHEN-004  فضای کار آشپزخانه دیواری

HIGH  ARCH-RES-KITCHEN-005  ممنوعیت عبور الزامی از آشپزخانه

HIGH  ARCH-RES-BATHROOM-001  ابعاد فضای بهداشتی مسکونی معمول

HIGH  ARCH-RES-BATHROOM-002  ارتفاع فضای بهداشتی مسکونی

HIGH  ARCH-RES-WC-001  پیش‌بینی توالت فرنگی برای واحد بزرگ

HIGH  ARCH-RES-DAYLIGHT-001  نور و هوای آشپزخانه مستقل

HIGH  ARCH-RES-DAYLIGHT-002  سطح شیشه اتاق اقامت

HIGH  ARCH-RES-DAYLIGHT-003  پنجره با لبه بالایی کوتاه

HIGH  ARCH-RES-DAYLIGHT-004  عمق قابل قبول نورگیری

HIGH  ARCH-RES-DAYLIGHT-005  نورگیری زیر ایوان

HIGH  ARCH-RES-DAYLIGHT-006  حیاط بین دو تصرف مسکونی

HIGH  ARCH-RES-DAYLIGHT-007  حیاط مشترک آشپزخانه و اقامت

HIGH  ARCH-RES-VENTILATION-001  بازشوی تهویه آشپزخانه

HIGH  ARCH-RES-VENTILATION-002  هود به هوای آزاد

CRITICAL  ARCH-RES-ACCESSIBILITY-001  فضای بهداشتی قابل استفاده معلول

# Dedicated Code 246 five percent decision

Source concept: حداقل 5 درصد واحدهای مسکونی. The prepared machine candidate stores raw_required_units = total_residential_units × 0.05. No integer conversion is active.

| A | CEILING_TO_NEXT_INTEGER |
| --- | --- |
| B | ANOTHER_AUTHORITY_BASED_METHOD — explicit authority basis required |
| C | NEEDS_MORE_EVIDENCE |
| Reviewer selection |  |
| Authority basis |  |
| Comments |  |

Floor or truncation is not an available default. Any alternate interpretation requires an authoritative basis.

# Dedicated M3 M4 Group 8 stair and egress review

| M4 Group 8 | Residential building height >23 m or floor count >7. |
| --- | --- |
| M4 stair geometry | Physical stair geometry remains distinct from fire egress logic. |
| M3 exit count | Two remote exits unless every condition of a specific exception is met. |
| M3 stair capacity | Occupant load and sprinkler-dependent width coefficients. |
| M3 enclosure and continuity | Explicit evidence required; missing status returns INPUT_REQUIRED. |
| One stair exception | The mapped residential exception is limited to <=6 storeys and <=23 m. |
| Code 246 | Accessible route and stair requirements apply only in their qualified branch. |
| Required confirmation | Group 8 cannot inherit an inapplicable lower-group one-stair exception. |
| Reviewer decision |  |
| Reviewer comments |  |

# Individual rule cards

# Rule 1  ARCH-RES-CIRCULATION-001

عرض راهروی داخلی واحد

| Risk | HIGH |
| --- | --- |
| Plain language rule | پهنای الزامی حداقل ۰٫۹۰ متر؛ الزامات خروج و دسترس‌پذیری جداگانه کنترل شوند. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Residential internal corridor or straight internal stair |
| Required inputs | corridor_width_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-6 |
| Page | printed 85 ¦ PDF 99 |
| Source text or precise summary | پهنای الزامی حداقل ۰٫۹۰ متر؛ الزامات خروج و دسترس‌پذیری جداگانه کنترل شوند. |
| Exceptions | None recorded |
| Cross-code dependencies | IR-M3, IR-ACCESS |
| Machine interpretation | {"field":"corridor_width_m","op":">=","value":0.9,"unit":"m"} |
| PASS example | Applicable project with complete authoritative inputs satisfying >= 0.9 m |
| FAIL example | Applicable project with complete authoritative inputs violating >= 0.9 m |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential internal corridor or straight internal stair; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: corridor_width_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 2  ARCH-RES-STAIRS-001

پلهٔ ساختمان مسکونی گروه ۱ تا ۳

| Risk | CRITICAL |
| --- | --- |
| Plain language rule | عرض مفید پله مستقیم حداقل ۰٫۹۰ و پله دارای گردش یا پاگرد حداقل ۱٫۱۰ متر. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Residential building group 1,2,3 established by authority |
| Required inputs | building_group, stair_type, stair_clear_width_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-3 |
| Page | printed 85 ¦ PDF 99 |
| Source text or precise summary | عرض مفید پله مستقیم حداقل ۰٫۹۰ و پله دارای گردش یا پاگرد حداقل ۱٫۱۰ متر. |
| Exceptions | None recorded |
| Cross-code dependencies | IR-M3, IR-ACCESS |
| Machine interpretation | {"review_predicate":{"inputs":["building_group","stair_type","stair_clear_width_m"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential building group 1,2,3 established by authority; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: building_group, stair_type, stair_clear_width_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 3  ARCH-RES-STAIRS-002

پلهٔ ساختمان مسکونی گروه ۴ تا ۷

| Risk | CRITICAL |
| --- | --- |
| Plain language rule | عرض مفید پله مستقیم حداقل ۱٫۱۰ و عرض قفسهٔ پلکان دارای پاگرد حداقل ۲٫۴۰ متر. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Residential building group 4,5,6,7 established by authority |
| Required inputs | building_group, stair_type, stair_clear_width_m, stair_enclosure_width_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-4 |
| Page | printed 85 ¦ PDF 99 |
| Source text or precise summary | عرض مفید پله مستقیم حداقل ۱٫۱۰ و عرض قفسهٔ پلکان دارای پاگرد حداقل ۲٫۴۰ متر. |
| Exceptions | None recorded |
| Cross-code dependencies | IR-M3, IR-ACCESS |
| Machine interpretation | {"review_predicate":{"inputs":["building_group","stair_type","stair_clear_width_m","stair_enclosure_width_m"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential building group 4,5,6,7 established by authority; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: building_group, stair_type, stair_clear_width_m, stair_enclosure_width_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 4  ARCH-RES-STAIRS-003

پلهٔ ساختمان مسکونی گروه ۸

| Risk | CRITICAL |
| --- | --- |
| Plain language rule | برای ساختمان مسکونی گروه ۸، هندسه پله مبحث چهارم و ظرفیت، تعداد، دوربندی و پیوستگی خروج مبحث سوم باید مستقل و هم‌زمان احراز شوند. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | building_use == RESIDENTIAL AND (building_height_m > 23 OR floor_count > 7) |
| Required inputs | building_use, building_height_m, floor_count, gross_residential_area_m2, sprinkler_status, exit_count, stair_count, stair_widths_cm, tread_depth_cm, riser_height_cm, headroom_cm, exit_enclosure_status, exit_continuity_status |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-5 |
| Page | printed 85 ¦ PDF 99 |
| Source text or precise summary | M4 4-7-1-1-5 defers group-8 stair dimensions to occupant population and M3 means-of-egress requirements. M3 3-6-11-2-2 requires two remote exits for apartments unless narrow exceptions apply; M3 3-6-4-3 and 3-6-6 provide stair geometry and capacity. |
| Exceptions | M3 3-6-11-2-3 through 3-6-11-2-5 only when every stated condition is met; group-8 height/storey conditions do not qualify for the <=6-storey, <=23 m single-stair exceptions. |
| Cross-code dependencies | IR-M3, IR-ACCESS |
| Machine interpretation | {"applicability":{"all":["building_use == RESIDENTIAL"],"any":["building_height_m > 23","floor_count > 7"],"missing_input":"INPUT_REQUIRED","false_result":"NOT_APPLICABLE"},"m3_occupant_load":{"formula":"gross_residential_area_m2 / 18.6","unit":"person","source_clause":"3-6-5-1","source_page":{"printed":99,"pdf":115}},"required_exit_count":{"operator":">=","value":2,"source_clause":"3-6-11-2-2","single_exit_exceptions":["3-6-11-2-3","3-6-11-2-4","3-6-11-2-5"],"group8_exception_eligibility":false,"reason":"Single-stair apartment exceptions are capped at six storeys and 23 m; group 8 exceeds that scope."},"stair_capacity":{"unsprinklered_mm_per_person":8,"sprinklered_mm_per_person":5,"source_clause":"3-6-6-1","source_page":{"printed":102,"pdf":118},"unknown_sprinkler":"INPUT_REQUIRED"},"stair_geometry":{"minimum_clear_width_cm":110,"conditional_minimum_width_cm":90,"conditional_width_scope":"all floors using that stair have fewer than 50 occupants","minimum_headroom_cm":205,"minimum_tread_cm":28,"riser_cm":{"minimum":10,"maximum":18},"tread_plus_two_risers_cm":{"minimum":63,"maximum":64},"source_clauses":["3-6-4-3-2","3-6-4-3-3","3-6-4-3-4"]},"egress_authority":{"requires":["exit_enclosure_status","exit_continuity_status"],"source_clauses":["3-6-3-3-3","3-6-2-16","3-6-11-2"],"missing_input":"INPUT_REQUIRED"},"combined_result":{"all_applicable_requirements_must_pass":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Group-8 apartment building with complete occupant/load/fire inputs, at least two independent exits, sufficient stair capacity, compliant stair geometry, enclosure and continuity. |
| FAIL example | Applicable group-8 building with one exit or insufficient stair width/geometry. |
| Boundary example | 23.00 m / 7 storeys remains below group-8 trigger; >23.00 m or >7 storeys enters the group-8 branch. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential group 8: building height > 23 m, with the M4 floor-count branch (>7 storeys) retained as an independently tested trigger.; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: building_use, building_height_m, floor_count, gross_residential_area_m2, sprinkler_status, exit_count, stair_count, stair_widths_cm, tread_depth_cm, riser_height_cm, headroom_cm, exit_enclosure_status, exit_continuity_status. |
| Current unresolved interpretation | Confirm the combined M4/M3/Code 246 interface and exclusion of lower-group one-stair exceptions. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 5  ARCH-RES-EGRESS-001

بازشدن در خروج از داخل

| Risk | CRITICAL |
| --- | --- |
| Plain language rule | بازکردن در از داخل ساده و فوری باشد؛ قفل یا مانع نباید خروج به‌موقع را مختل کند. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Doors on access/egress paths; explicit listed exceptions apply |
| Required inputs | door_hardware, egress_path, exception_authority |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-5-1-5-4 |
| Page | printed 47 ¦ PDF 61 |
| Source text or precise summary | بازکردن در از داخل ساده و فوری باشد؛ قفل یا مانع نباید خروج به‌موقع را مختل کند. |
| Exceptions | None recorded |
| Cross-code dependencies | IR-M3 |
| Machine interpretation | {"review_predicate":{"inputs":["door_hardware","egress_path","exception_authority"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Doors on access/egress paths; explicit listed exceptions apply; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: door_hardware, egress_path, exception_authority. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 6  ARCH-RES-DOOR-001

در ورودی اتاق اقامت

| Risk | HIGH |
| --- | --- |
| Plain language rule | پهنای مفید حداقل۰٫۸۰ متر و ارتفاع مفید۲٫۰۵ متر؛ لنگه فعال در دولنگه نیز حداقل۰٫۸۰ متر. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Habitable room entry; specific accessible/egress provisions also apply |
| Required inputs | door_clear_width_m, door_clear_height_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-5-2-8-2 |
| Page | printed 55 ¦ PDF 69 |
| Source text or precise summary | پهنای مفید حداقل۰٫۸۰ متر و ارتفاع مفید۲٫۰۵ متر؛ لنگه فعال در دولنگه نیز حداقل۰٫۸۰ متر. |
| Exceptions | None recorded |
| Cross-code dependencies | IR-M3, IR-ACCESS |
| Machine interpretation | {"all":[{"field":"door_clear_width_m","op":">=","value":0.8,"unit":"m"},{"field":"door_clear_height_m","op":">=","value":2.05,"unit":"m"}]} |
| PASS example | Applicable project with complete authoritative inputs satisfying >= 0.8 m, >= 2.05 m |
| FAIL example | Applicable project with complete authoritative inputs violating >= 0.8 m, >= 2.05 m |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Habitable room entry; specific accessible/egress provisions also apply; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: door_clear_width_m, door_clear_height_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 7  ARCH-RES-LIVING-001

اندازهٔ حداقل یک فضای اقامت اصلی

| Risk | HIGH |
| --- | --- |
| Plain language rule | حداقل یک فضای اقامت با مساحت ۱۲ مترمربع و عرض ۲٫۷۰ متر لازم است. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Residential unit area >=75m2 |
| Required inputs | unit_area_m2, largest_habitable_area_m2, largest_habitable_width_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-8 |
| Page | printed 85 ¦ PDF 99 |
| Source text or precise summary | حداقل یک فضای اقامت با مساحت ۱۲ مترمربع و عرض ۲٫۷۰ متر لازم است. |
| Exceptions | None recorded |
| Cross-code dependencies | unit_area_m2, largest_habitable_area_m2, largest_habitable_width_m |
| Machine interpretation | {"all":[{"field":"largest_habitable_area_m2","op":">=","value":12,"unit":"m2"},{"field":"largest_habitable_width_m","op":">=","value":2.7,"unit":"m"}]} |
| PASS example | Applicable project with complete authoritative inputs satisfying >= 12 m2, >= 2.7 m |
| FAIL example | Applicable project with complete authoritative inputs violating >= 12 m2, >= 2.7 m |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential unit area >=75m2; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: unit_area_m2, largest_habitable_area_m2, largest_habitable_width_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 8  ARCH-RES-LIVING-002

فضای اقامت واحد کوچک

| Risk | HIGH |
| --- | --- |
| Plain language rule | حداقل یک اتاق ۹ مترمربع و هیچ‌یک از ابعاد افقی آن کمتر از ۲٫۵ متر نباشد. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Residential unit area <75m2 |
| Required inputs | unit_area_m2, largest_habitable_area_m2, largest_habitable_width_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-8 |
| Page | printed 85 ¦ PDF 99 |
| Source text or precise summary | حداقل یک اتاق ۹ مترمربع و هیچ‌یک از ابعاد افقی آن کمتر از ۲٫۵ متر نباشد. |
| Exceptions | None recorded |
| Cross-code dependencies | unit_area_m2, largest_habitable_area_m2, largest_habitable_width_m |
| Machine interpretation | {"all":[{"field":"largest_habitable_area_m2","op":">=","value":9,"unit":"m2"},{"field":"largest_habitable_width_m","op":">=","value":2.5,"unit":"m"}]} |
| PASS example | Applicable project with complete authoritative inputs satisfying >= 9 m2, >= 2.5 m |
| FAIL example | Applicable project with complete authoritative inputs violating >= 9 m2, >= 2.5 m |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential unit area <75m2; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: unit_area_m2, largest_habitable_area_m2, largest_habitable_width_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 9  ARCH-RES-LIVING-003

مساحت عمومی اتاق اقامت

| Risk | HIGH |
| --- | --- |
| Plain language rule | حداقل مساحت ۶٫۵ مترمربع؛ حکم اختصاصی تصرف مقدم است. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Habitable room unless specific occupancy provisions supersede |
| Required inputs | room_area_m2 |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-5-2-2-1 |
| Page | printed 52 ¦ PDF 66 |
| Source text or precise summary | حداقل مساحت ۶٫۵ مترمربع؛ حکم اختصاصی تصرف مقدم است. |
| Exceptions | None recorded |
| Cross-code dependencies | room_area_m2 |
| Machine interpretation | {"field":"room_area_m2","op":">=","value":6.5,"unit":"m2"} |
| PASS example | Applicable project with complete authoritative inputs satisfying >= 6.5 m2 |
| FAIL example | Applicable project with complete authoritative inputs violating >= 6.5 m2 |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Habitable room unless specific occupancy provisions supersede; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: room_area_m2. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 10  ARCH-RES-BEDROOM-001

عرض سایر اتاق‌های اقامت مسکونی

| Risk | HIGH |
| --- | --- |
| Plain language rule | حداقل عرض سایر فضاهای اقامت ۲٫۴۰ متر است؛ مساحت طبق نوع استفاده کنترل شود. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Other habitable rooms in residential units; use-specific dimensions also apply |
| Required inputs | room_width_m, room_use |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-8 |
| Page | printed 85 ¦ PDF 99 |
| Source text or precise summary | حداقل عرض سایر فضاهای اقامت ۲٫۴۰ متر است؛ مساحت طبق نوع استفاده کنترل شود. |
| Exceptions | None recorded |
| Cross-code dependencies | room_width_m, room_use |
| Machine interpretation | {"field":"room_width_m","op":">=","value":2.4,"unit":"m"} |
| PASS example | Applicable project with complete authoritative inputs satisfying >= 2.4 m |
| FAIL example | Applicable project with complete authoritative inputs violating >= 2.4 m |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Other habitable rooms in residential units; use-specific dimensions also apply; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: room_width_m, room_use. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 11  ARCH-RES-KITCHEN-001

آشپزخانهٔ مستقل یا باز برای پخت‌وپز

| Risk | HIGH |
| --- | --- |
| Plain language rule | مساحت حداقل ۵٫۵۰ مترمربع؛ سطح آزاد خارج از قفسه‌بندی حداقل ۲٫۷۵ مترمربع. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Independent or open residential kitchen used only for cooking |
| Required inputs | kitchen_area_m2, kitchen_free_area_m2 |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-10 |
| Page | printed 86 ¦ PDF 100 |
| Source text or precise summary | مساحت حداقل ۵٫۵۰ مترمربع؛ سطح آزاد خارج از قفسه‌بندی حداقل ۲٫۷۵ مترمربع. |
| Exceptions | None recorded |
| Cross-code dependencies | kitchen_area_m2, kitchen_free_area_m2 |
| Machine interpretation | {"all":[{"field":"kitchen_area_m2","op":">=","value":5.5,"unit":"m2"},{"field":"kitchen_free_area_m2","op":">=","value":2.75,"unit":"m2"}]} |
| PASS example | Applicable project with complete authoritative inputs satisfying >= 5.5 m2, >= 2.75 m2 |
| FAIL example | Applicable project with complete authoritative inputs violating >= 5.5 m2, >= 2.75 m2 |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Independent or open residential kitchen used only for cooking; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: kitchen_area_m2, kitchen_free_area_m2. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 12  ARCH-RES-KITCHEN-002

آشپزخانه همراه محل صرف غذا

| Risk | HIGH |
| --- | --- |
| Plain language rule | حداقل مساحت ۷٫۵۰ مترمربع. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Residential independent/open kitchen for cooking and eating |
| Required inputs | kitchen_area_m2 |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-11 |
| Page | printed 86 ¦ PDF 100 |
| Source text or precise summary | حداقل مساحت ۷٫۵۰ مترمربع. |
| Exceptions | None recorded |
| Cross-code dependencies | kitchen_area_m2 |
| Machine interpretation | {"field":"kitchen_area_m2","op":">=","value":7.5,"unit":"m2"} |
| PASS example | Applicable project with complete authoritative inputs satisfying >= 7.5 m2 |
| FAIL example | Applicable project with complete authoritative inputs violating >= 7.5 m2 |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential independent/open kitchen for cooking and eating; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: kitchen_area_m2. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 13  ARCH-RES-KITCHEN-003

عرض آشپزخانه

| Risk | HIGH |
| --- | --- |
| Plain language rule | حداقل عرض ۱٫۸۰ متر؛ برای پخت‌وپز همراه صرف غذا ۲٫۱۵ متر. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Independent/open residential kitchen |
| Required inputs | kitchen_use, kitchen_width_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-12 |
| Page | printed 86 ¦ PDF 100 |
| Source text or precise summary | حداقل عرض ۱٫۸۰ متر؛ برای پخت‌وپز همراه صرف غذا ۲٫۱۵ متر. |
| Exceptions | None recorded |
| Cross-code dependencies | kitchen_use, kitchen_width_m |
| Machine interpretation | {"review_predicate":{"inputs":["kitchen_use","kitchen_width_m"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Independent/open residential kitchen; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: kitchen_use, kitchen_width_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 14  ARCH-RES-KITCHEN-004

فضای کار آشپزخانه دیواری

| Risk | HIGH |
| --- | --- |
| Plain language rule | فضای کار آزاد در تمام طول از لبه قفسه‌ها حداقل ۱٫۱۰ متر و طول آشپزخانه حداقل ۳ متر. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Wall kitchen in residential unit |
| Required inputs | work_aisle_width_m, wall_kitchen_length_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-13 |
| Page | printed 86 ¦ PDF 100 |
| Source text or precise summary | فضای کار آزاد در تمام طول از لبه قفسه‌ها حداقل ۱٫۱۰ متر و طول آشپزخانه حداقل ۳ متر. |
| Exceptions | None recorded |
| Cross-code dependencies | work_aisle_width_m, wall_kitchen_length_m |
| Machine interpretation | {"all":[{"field":"work_aisle_width_m","op":">=","value":1.1,"unit":"m"},{"field":"wall_kitchen_length_m","op":">=","value":3,"unit":"m"}]} |
| PASS example | Applicable project with complete authoritative inputs satisfying >= 1.1 m, >= 3 m |
| FAIL example | Applicable project with complete authoritative inputs violating >= 1.1 m, >= 3 m |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Wall kitchen in residential unit; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: work_aisle_width_m, wall_kitchen_length_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 15  ARCH-RES-KITCHEN-005

ممنوعیت عبور الزامی از آشپزخانه

| Risk | HIGH |
| --- | --- |
| Plain language rule | دسترسی فضای اقامت از آشپزخانه نباشد؛ استثنای واحد دو فضایی فقط با ورودی فعال و نور و تهویه مستقل بررسی شود. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Access to workplace/habitable room |
| Required inputs | access_graph, habitable_count, independent_openings |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-5-5-1-2 |
| Page | printed 58 ¦ PDF 72 |
| Source text or precise summary | دسترسی فضای اقامت از آشپزخانه نباشد؛ استثنای واحد دو فضایی فقط با ورودی فعال و نور و تهویه مستقل بررسی شود. |
| Exceptions | None recorded |
| Cross-code dependencies | access_graph, habitable_count, independent_openings |
| Machine interpretation | {"review_predicate":{"inputs":["access_graph","habitable_count","independent_openings"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Access to workplace/habitable room; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: access_graph, habitable_count, independent_openings. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 16  ARCH-RES-BATHROOM-001

ابعاد فضای بهداشتی مسکونی معمول

| Risk | HIGH |
| --- | --- |
| Plain language rule | حداقل عرض ۱ متر و طول ۱٫۲۰ متر؛ برای دوش مستقل با رختکن داخل فضا حداقل یکی از ابعاد ۱٫۵۰ متر. ادامه و استثنای سرویس ترکیبی در صفحه بعد بررسی شود. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Separate residential sanitary space without mandatory accessible designation |
| Required inputs | accessible_required, sanitary_width_m, sanitary_length_m, shower_dressing_inside |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-18 |
| Page | printed 86 ¦ PDF 100 |
| Source text or precise summary | حداقل عرض ۱ متر و طول ۱٫۲۰ متر؛ برای دوش مستقل با رختکن داخل فضا حداقل یکی از ابعاد ۱٫۵۰ متر. ادامه و استثنای سرویس ترکیبی در صفحه بعد بررسی شود. |
| Exceptions | None recorded |
| Cross-code dependencies | accessible_required, sanitary_width_m, sanitary_length_m, shower_dressing_inside |
| Machine interpretation | {"review_predicate":{"inputs":["accessible_required","sanitary_width_m","sanitary_length_m","shower_dressing_inside"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Separate residential sanitary space without mandatory accessible designation; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: accessible_required, sanitary_width_m, sanitary_length_m, shower_dressing_inside. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 17  ARCH-RES-BATHROOM-002

ارتفاع فضای بهداشتی مسکونی

| Risk | HIGH |
| --- | --- |
| Plain language rule | ارتفاع حداقل ۲٫۲۰ متر در ۸۰ درصد سطح الزامی؛ زیر سقف شیب‌دار کوتاه‌ترین قسمت حداقل ۲٫۰۵ متر. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Residential sanitary space |
| Required inputs | clear_height_m, height_coverage_fraction, sloped_roof_min_height_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-19 |
| Page | printed 87 ¦ PDF 101 |
| Source text or precise summary | ارتفاع حداقل ۲٫۲۰ متر در ۸۰ درصد سطح الزامی؛ زیر سقف شیب‌دار کوتاه‌ترین قسمت حداقل ۲٫۰۵ متر. |
| Exceptions | None recorded |
| Cross-code dependencies | clear_height_m, height_coverage_fraction, sloped_roof_min_height_m |
| Machine interpretation | {"review_predicate":{"inputs":["clear_height_m","height_coverage_fraction","sloped_roof_min_height_m"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential sanitary space; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: clear_height_m, height_coverage_fraction, sloped_roof_min_height_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 18  ARCH-RES-WC-001

پیش‌بینی توالت فرنگی برای واحد بزرگ

| Risk | HIGH |
| --- | --- |
| Plain language rule | فضا و تأسیسات آب و فاضلاب لازم برای نصب حداقل یک توالت فرنگی فراهم شود. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Residential unit area >=75m2 |
| Required inputs | unit_area_m2, wc_reservation, water_port, waste_port |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-17 |
| Page | printed 86 ¦ PDF 100 |
| Source text or precise summary | فضا و تأسیسات آب و فاضلاب لازم برای نصب حداقل یک توالت فرنگی فراهم شود. |
| Exceptions | None recorded |
| Cross-code dependencies | unit_area_m2, wc_reservation, water_port, waste_port |
| Machine interpretation | {"review_predicate":{"inputs":["unit_area_m2","wc_reservation","water_port","waste_port"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential unit area >=75m2; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: unit_area_m2, wc_reservation, water_port, waste_port. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 19  ARCH-RES-DAYLIGHT-001

نور و هوای آشپزخانه مستقل

| Risk | HIGH |
| --- | --- |
| Plain language rule | نور و تهویه طبیعی مستقل؛ در واحد زیر ۷۵ مترمربع اگر دورترین نقطه آشپزخانه از پنجره فضای مجاور کمتر از ۷ متر باشد استثنا بررسی شود. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Independent residential kitchen |
| Required inputs | unit_area_m2, kitchen_type, distance_to_adjacent_window_m, natural_light, natural_ventilation |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-14 |
| Page | printed 86 ¦ PDF 100 |
| Source text or precise summary | نور و تهویه طبیعی مستقل؛ در واحد زیر ۷۵ مترمربع اگر دورترین نقطه آشپزخانه از پنجره فضای مجاور کمتر از ۷ متر باشد استثنا بررسی شود. |
| Exceptions | None recorded |
| Cross-code dependencies | unit_area_m2, kitchen_type, distance_to_adjacent_window_m, natural_light, natural_ventilation |
| Machine interpretation | {"review_predicate":{"inputs":["unit_area_m2","kitchen_type","distance_to_adjacent_window_m","natural_light","natural_ventilation"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Independent residential kitchen; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: unit_area_m2, kitchen_type, distance_to_adjacent_window_m, natural_light, natural_ventilation. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 20  ARCH-RES-DAYLIGHT-002

سطح شیشه اتاق اقامت

| Risk | HIGH |
| --- | --- |
| Plain language rule | حداقل نسبت شیشه به کف ۱/۸؛ اگر پنجره‌ها تنها در یک دیوار و عمق بیش از ۴٫۵۰ متر باشد ۱/۷. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Habitable room; chapter4-6 and window-head exception3 apply |
| Required inputs | floor_area_m2, glazing_area_m2, single_window_wall, room_depth_m, window_head_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-5-2-3-2 |
| Page | printed 53 ¦ PDF 67 |
| Source text or precise summary | حداقل نسبت شیشه به کف ۱/۸؛ اگر پنجره‌ها تنها در یک دیوار و عمق بیش از ۴٫۵۰ متر باشد ۱/۷. |
| Exceptions | None recorded |
| Cross-code dependencies | floor_area_m2, glazing_area_m2, single_window_wall, room_depth_m, window_head_m |
| Machine interpretation | {"review_predicate":{"inputs":["floor_area_m2","glazing_area_m2","single_window_wall","room_depth_m","window_head_m"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Habitable room; chapter4-6 and window-head exception3 apply; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: floor_area_m2, glazing_area_m2, single_window_wall, room_depth_m, window_head_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 21  ARCH-RES-DAYLIGHT-003

پنجره با لبه بالایی کوتاه

| Risk | HIGH |
| --- | --- |
| Plain language rule | نسبت شیشه شفاف به کف حداقل ۱/۶؛ برای پنجره‌های یک‌طرفه و عمق بیش از۴٫۵۰ متر حداقل۱/۵. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Window head below2.10m excluding sloped-roof light |
| Required inputs | floor_area_m2, glazing_area_m2, window_head_m, single_window_wall, room_depth_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-5-2-3-3 |
| Page | printed 53 ¦ PDF 67 |
| Source text or precise summary | نسبت شیشه شفاف به کف حداقل ۱/۶؛ برای پنجره‌های یک‌طرفه و عمق بیش از۴٫۵۰ متر حداقل۱/۵. |
| Exceptions | None recorded |
| Cross-code dependencies | floor_area_m2, glazing_area_m2, window_head_m, single_window_wall, room_depth_m |
| Machine interpretation | {"review_predicate":{"inputs":["floor_area_m2","glazing_area_m2","window_head_m","single_window_wall","room_depth_m"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Window head below2.10m excluding sloped-roof light; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: floor_area_m2, glazing_area_m2, window_head_m, single_window_wall, room_depth_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 22  ARCH-RES-DAYLIGHT-004

عمق قابل قبول نورگیری

| Risk | HIGH |
| --- | --- |
| Plain language rule | حداکثر عمق قابل قبول نورگیری از یک پنجره ۷ متر. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Habitable room served by a window |
| Required inputs | daylight_depth_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-5-2-8-3 |
| Page | printed 55 ¦ PDF 69 |
| Source text or precise summary | حداکثر عمق قابل قبول نورگیری از یک پنجره ۷ متر. |
| Exceptions | None recorded |
| Cross-code dependencies | daylight_depth_m |
| Machine interpretation | {"field":"daylight_depth_m","op":"<=","value":7,"unit":"m"} |
| PASS example | Applicable project with complete authoritative inputs satisfying <= 7 m |
| FAIL example | Applicable project with complete authoritative inputs violating <= 7 m |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Habitable room served by a window; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: daylight_depth_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 23  ARCH-RES-DAYLIGHT-005

نورگیری زیر ایوان

| Risk | HIGH |
| --- | --- |
| Plain language rule | عمق نورگیری از لبه خارجی ایوان محسوب شود. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Window below balcony/terrace projection |
| Required inputs | balcony_outer_edge, room_daylight_geometry |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-5-2-8-4 |
| Page | printed 55 ¦ PDF 69 |
| Source text or precise summary | عمق نورگیری از لبه خارجی ایوان محسوب شود. |
| Exceptions | None recorded |
| Cross-code dependencies | balcony_outer_edge, room_daylight_geometry |
| Machine interpretation | {"review_predicate":{"inputs":["balcony_outer_edge","room_daylight_geometry"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Window below balcony/terrace projection; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: balcony_outer_edge, room_daylight_geometry. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 24  ARCH-RES-DAYLIGHT-006

حیاط بین دو تصرف مسکونی

| Risk | HIGH |
| --- | --- |
| Plain language rule | فاصله دیوارهای پنجره‌های مقابل حداقل۶ متر. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Land area >200m2; main habitable rooms of two independent units use same internal court |
| Required inputs | land_area_m2, shared_court, opposing_room_uses, window_wall_distance_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-24 |
| Page | printed 87 ¦ PDF 101 |
| Source text or precise summary | فاصله دیوارهای پنجره‌های مقابل حداقل۶ متر. |
| Exceptions | None recorded |
| Cross-code dependencies | land_area_m2, shared_court, opposing_room_uses, window_wall_distance_m |
| Machine interpretation | {"review_predicate":{"inputs":["land_area_m2","shared_court","opposing_room_uses","window_wall_distance_m"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Land area >200m2; main habitable rooms of two independent units use same internal court; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: land_area_m2, shared_court, opposing_room_uses, window_wall_distance_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 25  ARCH-RES-DAYLIGHT-007

حیاط مشترک آشپزخانه و اقامت

| Risk | HIGH |
| --- | --- |
| Plain language rule | فاصله دیوارهای پنجره‌های مقابل حداقل۴ متر. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Independent units: kitchen/habitable or two kitchens use same internal court |
| Required inputs | shared_court, opposing_room_uses, window_wall_distance_m |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-25 |
| Page | printed 87 ¦ PDF 101 |
| Source text or precise summary | فاصله دیوارهای پنجره‌های مقابل حداقل۴ متر. |
| Exceptions | None recorded |
| Cross-code dependencies | shared_court, opposing_room_uses, window_wall_distance_m |
| Machine interpretation | {"review_predicate":{"inputs":["shared_court","opposing_room_uses","window_wall_distance_m"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Independent units: kitchen/habitable or two kitchens use same internal court; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: shared_court, opposing_room_uses, window_wall_distance_m. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 26  ARCH-RES-VENTILATION-001

بازشوی تهویه آشپزخانه

| Risk | HIGH |
| --- | --- |
| Plain language rule | حداقل سطح بازشوی تهویه یک شانزدهم مساحت کف آشپزخانه. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Kitchen serving habitable rooms |
| Required inputs | kitchen_area_m2, vent_opening_area_m2 |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-15 |
| Page | printed 86 ¦ PDF 100 |
| Source text or precise summary | حداقل سطح بازشوی تهویه یک شانزدهم مساحت کف آشپزخانه. |
| Exceptions | None recorded |
| Cross-code dependencies | kitchen_area_m2, vent_opening_area_m2 |
| Machine interpretation | {"field":"vent_opening_area_m2","op":">=","rhs_field":"kitchen_area_m2","factor":0.0625,"unit":"m2"} |
| PASS example | Applicable project with complete authoritative inputs satisfying factor 0.0625 |
| FAIL example | Applicable project with complete authoritative inputs violating factor 0.0625 |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Kitchen serving habitable rooms; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: kitchen_area_m2, vent_opening_area_m2. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 27  ARCH-RES-VENTILATION-002

هود به هوای آزاد

| Risk | HIGH |
| --- | --- |
| Plain language rule | هود با کانال مستقل مکش به خارج ساختمان مرتبط شود. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Kitchen ventilation requires over-cooktop extraction |
| Required inputs | extract_required, hood_port, independent_exhaust_route |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-5-5-3-3 |
| Page | printed 59 ¦ PDF 73 |
| Source text or precise summary | هود با کانال مستقل مکش به خارج ساختمان مرتبط شود. |
| Exceptions | None recorded |
| Cross-code dependencies | extract_required, hood_port, independent_exhaust_route |
| Machine interpretation | {"review_predicate":{"inputs":["extract_required","hood_port","independent_exhaust_route"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Kitchen ventilation requires over-cooktop extraction; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: extract_required, hood_port, independent_exhaust_route. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 28  ARCH-RES-BALCONY-001

فضای نیمه‌باز اختصاصی

| Risk | MEDIUM |
| --- | --- |
| Plain language rule | حداقل یک فضای نیمه‌باز اختصاصی مطابق ۴-۵-۷ لازم است؛ اندازه بدون منبع فرض نشود. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Apartment/unit without direct adjacency and access to open space |
| Required inputs | direct_open_space_access, private_semiopen_space, m4_5_7_result |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-22 |
| Page | printed 87 ¦ PDF 101 |
| Source text or precise summary | حداقل یک فضای نیمه‌باز اختصاصی مطابق ۴-۵-۷ لازم است؛ اندازه بدون منبع فرض نشود. |
| Exceptions | None recorded |
| Cross-code dependencies | direct_open_space_access, private_semiopen_space, m4_5_7_result |
| Machine interpretation | {"review_predicate":{"inputs":["direct_open_space_access","private_semiopen_space","m4_5_7_result"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Apartment/unit without direct adjacency and access to open space; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: direct_open_space_access, private_semiopen_space, m4_5_7_result. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 29  ARCH-RES-PRIVACY-001

دسترسی مستقل اتاق‌ها

| Risk | MEDIUM |
| --- | --- |
| Plain language rule | دسترسی اتاق اقامت مستقیماً از اتاق دیگر یا نشیمن عمومی نباشد مگر استثناهای صریح همین بند؛ دید مستقیم از نشیمن عمومی به خواب در هر حال مجاز نیست. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Residential habitable rooms; apply listed exceptions a-1,a-2,a-3 |
| Required inputs | access_graph, unit_area_m2, habitable_count, entry_buffer, sightlines |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-2.a |
| Page | printed 84 ¦ PDF 98 |
| Source text or precise summary | دسترسی اتاق اقامت مستقیماً از اتاق دیگر یا نشیمن عمومی نباشد مگر استثناهای صریح همین بند؛ دید مستقیم از نشیمن عمومی به خواب در هر حال مجاز نیست. |
| Exceptions | None recorded |
| Cross-code dependencies | access_graph, unit_area_m2, habitable_count, entry_buffer, sightlines |
| Machine interpretation | {"review_predicate":{"inputs":["access_graph","unit_area_m2","habitable_count","entry_buffer","sightlines"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential habitable rooms; apply listed exceptions a-1,a-2,a-3; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: access_graph, unit_area_m2, habitable_count, entry_buffer, sightlines. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 30  ARCH-RES-PRIVACY-002

دید به سرویس بهداشتی

| Risk | MEDIUM |
| --- | --- |
| Plain language rule | دید به داخل سرویس ممنوع؛ پیش‌ورودی دارای روشویی فقط با حفظ حرکت و دسترسی به سایر تجهیزات مجاز است. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Combined sanitary facility visible from living/hall |
| Required inputs | sanitary_sightlines, ante_room, fixture_access |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-2.p |
| Page | printed 84 ¦ PDF 98 |
| Source text or precise summary | دید به داخل سرویس ممنوع؛ پیش‌ورودی دارای روشویی فقط با حفظ حرکت و دسترسی به سایر تجهیزات مجاز است. |
| Exceptions | None recorded |
| Cross-code dependencies | sanitary_sightlines, ante_room, fixture_access |
| Machine interpretation | {"review_predicate":{"inputs":["sanitary_sightlines","ante_room","fixture_access"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Combined sanitary facility visible from living/hall; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: sanitary_sightlines, ante_room, fixture_access. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 31  ARCH-RES-PRIVACY-003

دید به محل کار آشپزخانه

| Risk | MEDIUM |
| --- | --- |
| Plain language rule | از نشیمن عمومی یا سالن دید مستقیم به قسمت‌های اصلی پخت‌وپز و کار آشپزخانه نباشد. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | Residential unit area >=75m2, wall/open kitchen |
| Required inputs | unit_area_m2, kitchen_type, living_kitchen_sightlines |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-7-1-1-2.th |
| Page | printed 84 ¦ PDF 98 |
| Source text or precise summary | از نشیمن عمومی یا سالن دید مستقیم به قسمت‌های اصلی پخت‌وپز و کار آشپزخانه نباشد. |
| Exceptions | None recorded |
| Cross-code dependencies | unit_area_m2, kitchen_type, living_kitchen_sightlines |
| Machine interpretation | {"review_predicate":{"inputs":["unit_area_m2","kitchen_type","living_kitchen_sightlines"],"expected":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | Applicable project with complete authoritative inputs satisfying qualitative predicate |
| FAIL example | Applicable project with complete authoritative inputs violating qualitative predicate |
| Boundary example | Evaluate exact threshold and immediately below/above values where numeric; otherwise exact predicate boundary. |
| NOT_APPLICABLE example | Project/object does not satisfy: Residential unit area >=75m2, wall/open kitchen; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: unit_area_m2, kitchen_type, living_kitchen_sightlines. |
| Current unresolved interpretation | None beyond licensed professional confirmation of the prepared interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |

# Rule 32  ARCH-RES-ACCESSIBILITY-001

فضای بهداشتی قابل استفاده معلول

| Risk | CRITICAL |
| --- | --- |
| Plain language rule | در هر واحد مسکونی قابل دسترس، حداقل یک فضای بهداشتی باید هم‌زمان با بند ۳-۲-۳-۸ ضابطه ۲۴۶، ارجاع آن به ۳-۱-۱-۱۲ و حداقل ۱٫۵۰×۱٫۷۰ متر مبحث چهارم منطبق باشد. |
| Classification | MANDATORY_NATIONAL |
| Enforcement consequence | BLOCK |
| Applicable conditions | building_use == RESIDENTIAL AND accessible_residential_unit == true |
| Required inputs | building_use, units_per_floor, floor_count, total_residential_units, fully_government_funded, local_elevator_requirement, accessible_residential_unit, accessible_sanitary_space_count, sanitary_dimensions_m, accessible_route_result |
| Authoritative source | مبحث چهارم، الزامات عمومی ساختمان |
| Edition | 1396 |
| Clause | 4-5-6-2-2 |
| Page | printed 61 ¦ PDF 75 |
| Source text or precise summary | Code 246 defines a residential complex as more than four units on one floor or more than eight units across multiple floors; 3-2-2-1 requires at least five percent accessible units and at least one in fully government-funded complexes under 20 units; 3-2-3-8 requires one sanitary space conforming to 3-1-1-12. |
| Exceptions | Five-percent integer conversion is not stated in the inspected Code 246 text and remains HUMAN_INTERPRETATION_REQUIRED.; The under-20 minimum-one-unit branch applies only to fully government-funded residential complexes.; Local elevator triggers are never inferred from floor count. |
| Cross-code dependencies | IR-ACCESS, IR-M4, IR-M3_ACCESSIBLE_EGRESS_WHEN_APPLICABLE |
| Machine interpretation | {"applicability":{"building_use":"RESIDENTIAL","accessible_residential_unit":true,"missing_input":"INPUT_REQUIRED","false_result":"NOT_APPLICABLE"},"residential_complex":{"any":["units_per_floor > 4","floor_count > 1 AND total_residential_units > 8"],"source":"Code 246 Chapter 1 definition","source_page":{"printed":7,"pdf":25},"missing_input":"INPUT_REQUIRED"},"accessible_unit_requirement":{"private_or_general_complex":{"calculation_raw":"total_residential_units * 0.05","integer_conversion":"HUMAN_INTERPRETATION_REQUIRED"},"fully_government_funded_under_20":{"when":"fully_government_funded == true AND total_residential_units < 20","minimum_accessible_units":1},"source_clause":"3-2-2-1","source_page":{"printed":91,"pdf":109}},"common_area_scope":{"local_elevator_required":{"when":"local_elevator_requirement == true","result":"elevator_and_common_areas_accessible"},"below_local_elevator_trigger":{"when":"local_elevator_requirement == false","result":"ground_floor_and_common_areas_accessible"},"unknown_local_elevator_requirement":"LOCAL_RULE_REQUIRED","source_clauses":["3-2-2-2","3-2-2-3"],"source_page":{"printed":92,"pdf":110}},"accessible_unit_dependencies":{"clauses":["3-2-3-1","3-2-3-2","3-2-3-3","3-2-3-4","3-2-3-5","3-2-3-6","3-2-3-7","3-2-3-8","3-2-3-9"],"shared_rules":["code246_entry","code246_corridor","code246_stair","code246_ramp","code246_elevator","code246_openings","code246_handrails","code246_sanitary","code246_controls"]},"sanitary_check":{"minimum_count":1,"m4_minimum_dimensions_m":[1.5,1.7],"requires_code246_clause":"3-2-3-8 -> 3-1-1-12","missing_input":"INPUT_REQUIRED","false_result":"FAIL"},"parking_scope":{"accessible_unit_minimum_spaces":1,"source_clause":"3-2-3-1","source_page":{"printed":93,"pdf":111},"public_parking_rule":"2-1-5-2","local_general_parking_count":"LOCAL_RULE_REQUIRED"},"result_logic":{"all_applicable_requirements_must_pass":true,"missing_input":"INPUT_REQUIRED","false_result":"FAIL","not_applicable_result":"NOT_APPLICABLE"}} |
| PASS example | An independently classified accessible residential unit contains at least one sanitary space meeting M4 dimensions and Code 246 clause 3-1-1-12. |
| FAIL example | An applicable accessible unit lacks a compliant sanitary space or route. |
| Boundary example | 4 units on one floor is not a residential complex; 5 is. Eight units across multiple floors is not; 9 is. Five-percent integer conversion remains a bounded interpretation question. |
| NOT_APPLICABLE example | Project/object does not satisfy: Accessible residential unit sanitary space: applies only after Code 246 residential-complex/accessibility classification or an authoritative project requirement establishes an accessible unit.; result must be NOT_APPLICABLE, never PASS. |
| INPUT_REQUIRED behavior | If any material applicability or evaluation input is absent, return INPUT_REQUIRED. Required inputs: building_use, units_per_floor, floor_count, total_residential_units, fully_government_funded, local_elevator_requirement, accessible_residential_unit, accessible_sanitary_space_count, sanitary_dimensions_m, accessible_route_result. |
| Current unresolved interpretation | Code 246 five-percent integer conversion requires licensed interpretation. |
| Proposed reviewer decision | No default. Reviewer must select one allowed decision. |

## Reviewer decision

☐ APPROVED     ☐ APPROVED_WITH_CHANGE     ☐ REJECTED     ☐ NEEDS_MORE_EVIDENCE

| Reviewer comments |  |
| --- | --- |
| Requested change or missing evidence |  |
| Source basis |  |
| Reviewer name |  |
| Professional role |  |
| License or registration |  |
| Review date |  |